# ECG Arrhythmia Classification using Scalogram-Based Deep Learning and BrainChip Akida

## 1. Project Objective

The objective of this project was to develop a lightweight ECG arrhythmia classification model suitable for deployment on BrainChip Akida hardware.

The study focused on:

- Converting ECG beats into image representations using Wavelet Scalograms.

- Incorporating RR interval information to improve classification performance.

- Evaluating both 5-class and 3-class classification approaches.

- Optimizing the model for quantization-aware training (QAT) and Akida deployment.

- Investigating pruning and sparsity techniques to improve hardware efficiency.

## 2. Dataset Preparation

### Source Dataset

MIT-BIH Arrhythmia Database was used as the primary dataset.

### Beat Extraction

For each annotated heartbeat:

- ECG beat window extracted around R-peak.

- Window sizes evaluated:

  - 200 samples

  - 250 samples

  - 300 samples

### Additional Features

- RR Internal features were extracted:

- Previous RR interval (rr_prev)

- Next RR interval (rr_next)

- Local normalized RR interval (rr_local)

- Average RR interval over the previous 10 beats (rr_avg)

The previous RR interval was computed as the time difference between the current and preceding R-peaks, while the next RR interval was calculated using the subsequent R-peak. A local average RR interval was estimated from the preceding 10 beats, and the current RR interval was normalized by this average to derive the local RR feature. These RR features were appended as additional rows to the scalogram representation, providing temporal context for downstream model training and inference.

## 3. Scalogram Generation

Continuous Wavelet Transform (CWT) was applied to convert ECG beats into time-frequency images.

- Scalogram Size: 32 × 32

- RR Feature Rows: 4

- Final Input Size: 36 × 32

## 4. Model Architecture

A lightweight Akida-compatible CNN architecture was designed.

### Key Components

- Conv2D Stem Layer

- Depthwise Separable Convolution Blocks

- Batch Normalization

- ReLU6 Activations

- Global Average Pooling

- Dense Classification Head

Design considerations:

- Small parameter count

- Akida compatibility

- Efficient quantization

## 5. Five-Class Classification Experiments

### Classes

| AAMI Class | Description |
| --- | --- |
| N | Normal |
| S | Supraventricular |
| V | Ventricular |
| F | Fusion |
| Q | Unknown |

### Baseline Observations

Strong performance:

- N class

- V class

Weak performance:

- S class

- F class

- Q class

Main challenge: The S-class contains multiple beat morphologies with significant overlap with normal beats.

## 6. Experiments Performed

### 6.1 Window Size Evaluation

#### Window Size = 200

Provided reasonable baseline performance.

#### Window Size = 250

Improved:

- Overall accuracy

- V-class recall

- F-class recall

Selected as final configuration.

Result:

✓ Improvement observed

### 6.2 RR Interval Features

Experiments:

- Scalogram only

- Scalogram + RR rows

Observation:

RR features improved rhythm-related discrimination.

Selected:

- 4 RR rows

Result:

✓ Improvement observed

### 6.3 Class Weight Tuning

Multiple class-weight configurations evaluated.

Best-performing configuration:

{
    0: 1.0,
    1: 8.0,
    2: 3.0,
    3: 8.0,
    4: 10.0
}

Observation:

Improved:

- S-class recall

- F-class recall

Trade-off:

- Slight reduction in precision.

Result:

✓ Improvement observed

### 6.4 Additional Dataset Integration (INCART)

Objective:

Increase minority-class representation.

Observation:

- Distribution mismatch observed.

- Limited improvement in classification performance.

- Introduced additional variability.

Result:

✗ No meaningful improvement

Recommendation:

MIT-BIH only.

## 7. Best Five-Class Results

### Floating Point Model

Accuracy ≈ 92%

Class-wise observations:

- Strong N performance

- Strong V performance

- Moderate F performance

- Weak S performance

- Q class extremely limited due to insufficient samples

### Quantized Model

Accuracy ≈ 93%

Quantization drop:

~-1%

## 8. Root Cause Analysis

Detailed subclass analysis performed.

### S-Class Breakdown

Subclasses analyzed:

- A

- J

- a

Observations:

#### A Beats

Majority of S-class samples.

Recall approximately: ~66%

Main source of classification errors.

#### J Beats

Moderate performance.

Recall approximately: ~49%

#### a Beats

Moderate performance.

Recall approximately: ~48%

Conclusion:

Most S-class errors originate from A-type atrial premature beats.

## 9. Transition to Three-Class Classification

Based on customer discussions and observed class overlap, a simplified three-class approach was evaluated.

### Classes

| Class | Included Beats |
| --- | --- |
| N | N, L, R |
| S | A, J, a |
| V | V |

Fusion and Unknown classes excluded.

## 10. Three-Class Results

### Best Floating Point Model

Accuracy:

~95-96%

Class Performance:

| Class | Precision | Recall |
| --- | --- | --- |
| N | Very High | Very High |
| S | Moderate | Strong Improvement |
| V | High | High |

Observation:

Significant improvement over five-class classification.

## 11. Quantization-Aware Training (QAT)

BrainChip quantization pipeline:

- Input Quantization = 8-bit

- Weight Quantization = 4-bit

- Activation Quantization = 4-bit

### Best Results

Window Size: 250

FP32 Accuracy:

~94.39%

QAT Accuracy:

~ 95.73%

Conclusion:

Model is highly quantization-friendly.

## 12. Sparsity and Pruning Experiments

### Activation Sparsity

Measured layer-wise activation sparsity.

Key observations:

- Early layers: 60–70%

- Deeper layers: >90%

This indicates strong natural sparsity suitable for Akida.

### Activity Regularization

L1/L2 activity regularization.

| Layer Name | Sparsity (% Zeros) |
| --- | --- |
| stem_conv | 93.91% |
| block1_sepconv | 72.70% |
| block2_sepconv | 94.75% |
| block3_sepconv | 78.05% |
| dense | 79.20% |
| dense_1 | 0.00% |

Observation:

Increased activation sparsity significantly.

## 13. Key Findings

### What Worked

✓ Window size 250

✓ Scalogram + RR features

✓ Class-weight tuning

✓ Lightweight Akida CNN

✓ Three-class formulation

✓ Quantization-aware training

✓ Weight pruning before QAT

✓ Activity regularization

✓ Preserving sparsity after Akida QAT

### What Did Not Work

✗ Additional INCART dataset

✗ Five-class formulation for reliable S-class performance

## 14. Final Recommendation

### Recommended Production Model

Configuration:

- Window Size: 250

- Scalogram: 32 × 32

- RR Rows: 4

- Input Size: 32 × 36

- Three-Class Classification

- Lightweight Separable CNN

- Quantization-Aware Training

### Expected Performance

FP32 Accuracy: ~94-95%

QAT Accuracy: ~95-96%

### Deployment Target

BrainChip Akida Hardware

This configuration provides the best balance between:

- Accuracy

- Model size

- Quantization robustness

- Hardware efficiency

- Deployment readiness

# 15. Best Model

| ## Accuracy | ## Inference Time | ## Latency of inference | ## Throughput |
| --- | --- | --- | --- |
| ## 96% | ## 59.47 s (49,280 beats) | ## 1.207 ms/sample | ## 828.67 beats/sec |

## Classification Performance

| ## Class | ## Precision | ## Recall | ## F1-Score |
| --- | --- | --- | --- |
| ## N | ## 0.98 | ## 0.97 | ## 0.98 |
| ## S | ## 0.63 | ## 0.71 | ## 0.67 |
| ## V | ## 0.85 | ## 0.88 | ## 0.85 |
| ## Macro Avg | ## 0.82 | ## 0.85 | ## 0.84 |
| ## Weighted Avg | ## 0.96 | ## 0.96 | ## 0.96 |

## Confusion Matrix

| ## Actual \ Predicted | ## N | ## S | ## V |
| --- | --- | --- | --- |
| ## N | ## 43,098 | ## 710 | ## 416 |
| ## S | ## 391 | ## 1298 | ## 148 |
| ## V | ## 345 | ## 52 | ## 2822 |

# Reference:

- Lightweight Deep Learning Architecture for Multi-Lead ECG Arrhythmia Detection

- Intra- and Interpatient ECG Heartbeat Classification Based on Multimodal Convolutional Neural Networ…

- A Tiny Matched Filter-Based CNN for Inter-Patient ECG Classification and Arrhythmia Detection at the…

- ECG Heartbeat Classification: A Deep Transferable Representation